# Chapter Four: Implementation

## 4.1 Introduction

This chapter documents the implementation of the IoT-enabled mobile-robot emergency-response system whose architecture and requirements were laid out in Chapter Three. The system is realised as a six-package ROS 2 workspace running against a custom Gazebo Sim hospital world. It detects a simulated patient emergency, plans a route from the nurse station to the patient’s bed using either an A* or an RRT* planner, drives the robot to the patient through dynamic crowds of moving humans, and records every timestamp needed to compare planner performance.

The implementation follows the experimental research approach motivated in Chapter Three (§3.3) and the build checklist in the project PRD §12. All artefacts referenced in this chapter exist in fyp_execution_kit/ and appear in the appendix file index.

## 4.2 Development Environment

| Component | Choice |
| --- | --- |
| Operating system | Ubuntu 24.04 LTS |
| ROS 2 distribution | ROS 2 Jazzy Jalisco |
| Simulator | Gazebo Sim Harmonic (gz sim) |
| Visualisation | RViz 2 |
| ROS ↔ Gazebo bridge | ros_gz_bridge, ros_gz_sim |
| Languages | Python 3.12 |
| Standard ROS libs | rclpy, nav_msgs, geometry_msgs, std_msgs, tf2_ros |
| Analysis libraries | csv, statistics, Matplotlib |
| Build system | colcon with ament_python |
| Editor | Visual Studio Code |
| Version control | Git, hosted on GitHub |

The full apt install command list for setting up a fresh Ubuntu 24.04 machine is given in Appendix B – Reproducibility.

## 4.3 System Module Implementation

The system is composed of five ROS 2 packages mapping one-to-one onto the architecture layers in Chapter Three §3.4. Each package exposes a single node, and the launch file emergency_launch/launch/emergency_sim.launch.py brings them up together with Gazebo Sim, the bridge, the static TF publisher, and RViz 2.

### 4.3.1 IoT Sensing Module — emergency_sensing

The IoT sensing module is realised as two nodes: sensor_simulator and alert_fusion (ros2_ws/src/emergency_sensing/emergency_sensing/). The sensor_simulator node models a wearable / environmental sensor stack that emits camera-fall, heart-rate, and motion-state signals alongside a synthetic camera image stream. Patient locations are loaded from the scenario YAML; on each run the node selects one location with a seed-controlled random choice, builds the sensor streams, and starts publishing after a configurable first_alert_delay_s.

The alert_fusion node subscribes to these sensor topics, applies simple thresholding and rule-based logic (e.g., fall detected + high heart rate + no motion), and publishes the consolidated alert on /emergency_alert.

The alert payload (std_msgs/String carrying JSON):

{
  "alert_id": "run17_alert1_84321",
  "event_type": "fall_detected",
  "patient_id": "patient_b",
  "severity": "high",
  "location": {"x": 12, "y": 9},
  "run_id": 17,
  "sequence": 0,
  "t_alert_ns": 1759230432123456789
}

The alert_id is the join key used by every downstream node to attach their per-event timestamps and outcomes back to a single trial row in trial_results.csv. The t_alert_ns value uses the simulation clock when the bridge publishes /clock, ensuring response-time measurements are not distorted by wall-clock drift inside Gazebo.

std_msgs/String carrying JSON was preferred over a custom .msg type in this iteration because it allowed the alert / route / status schemas to evolve without forcing a colcon build of an interface package between every change.

### 4.3.2 Fusion / Decision Module

Fusion and decision-making are handled by the alert_fusion node in emergency_sensing. This module consumes the raw simulated sensor streams and emits a single canonical incident when its rule set is met. The resulting message includes alert_id, event_type, and location, which the planner consumes directly. In a richer deployment, this module would be extended to a probabilistic or learning-based fusion approach; that future-work extension is captured in Chapter Five §5.7.

### 4.3.3 Path-Planning Module — path_planner

The path_planner node (ros2_ws/src/path_planner/path_planner/path_planner_node.py) implements both planners behind a single planner_mode parameter. Its grid model is loaded at start-up from the scenario YAML so the same authored layout drives both Gazebo (via the SDF walls) and the planner (via the planner_grid.obstacles list) — this prevents the planner from returning paths that walk through walls, which had been a problem in the earlier hard-coded grid (§4.7).

The planner maintains three obstacle sets: 1. Static obstacles: loaded once from the scenario YAML; these model walls and beds. 2. Event-triggered obstacles: added transiently when /replan_request fires; these model moving humans that caused a stall. 3. Human-driven obstacles: continuously updated from /dynamic_obstacles; these model live actor positions from Gazebo.

During path search, a cell is blocked if it belongs to any of the three sets or if it is out of bounds.

#### A* Implementation

A* is implemented over the 24 × 14 unit-cell grid with 4-connectivity and the Manhattan heuristic h(n) = |n.x − g.x| + |n.y − g.y|. The cost of each edge is uniformly 1, reflecting the 1 m grid spacing and the absence of slope or surface-cost data in the simulated hospital floor. The implementation uses a binary heap (heapq) for the open set and a set for the closed set, giving the textbook O((V + E) log V) time complexity. With 24 × 14 = 336 cells the worst-case search space is small and A* returns in well under a millisecond on the test machine.

Tie-breaking in the open heap follows the insertion order, ensuring consistent path selection across runs when costs are equal.

#### RRT* Implementation

RRT* is implemented with the same scenario grid as the implicit obstacle predicate, a 25 % goal-bias sampling rate, and a near() rewiring radius that scales as min(4.0, 2 √(log n / n) · max(W, H)) to keep the local neighbourhood small in dense regions. steer() projects each random sample onto the segment of length step_size (0.8 m by default) from the nearest tree node, and is_collision_free() discretises the segment into 0.5 m-spaced waypoints and rejects any whose grid cell is blocked. The planner declares the parameters rrt_max_iterations, rrt_step_size, and rrt_goal_sample_rate; their defaults (2000 / 0.8 / 0.25) were chosen during Day 5 tuning (§4.7) and are recorded in emergency_sim.launch.py.

Unlike A*, which is deterministic, RRT* is seeded from the seed parameter so that repeated runs with the same scenario and seed produce the same sequences of random samples and thus the same final paths. This allows fair head-to-head comparison.

#### Dynamic Re-Planning

Both planners support re-planning triggered by /replan_request. On a request the node injects a transient obstacle near the robot’s current grid cell (to model the moving human that caused the stall), increments a per-alert replan counter, and re-runs the active planner. After 6 replans for a single alert the system flags the trial as failed to prevent infinite loops.

### 4.3.4 Navigation and Control Module — robot_control

The robot_controller node (ros2_ws/src/robot_control/robot_control/robot_controller_node.py) follows the planned route as a list of metric waypoints. It uses a simple proportional controller: linear speed is min(v_max, K_v · d) where d is the distance to the next waypoint, and angular speed is K_ω · θ_e where θ_e is the heading error to the next waypoint, clipped to ±ω_max. When the heading error exceeds 1 rad the linear command is scaled to 20 % of its target, which lets the robot turn in place near sharp corners without overshoot. Arrival is declared when the Euclidean distance to the final waypoint falls under waypoint_tolerance (0.4 m).

Stall detection. A stall is detected when the robot has moved less than 5 cm over stall_timeout_s (2.5 s) while the most recent linear command exceeded stall_min_cmd_linear (0.05 m/s). When a stall fires the controller publishes a /replan_request payload containing the current grid cell of the robot, and the planner injects a transient obstacle at an adjacent cell before re-planning. A per-alert replan budget of 6 prevents the system from looping indefinitely; exhausting it ends the trial as a failure with reason replan_budget_exhausted.

The cmd_vel / odom topics are remapped at launch time to the bridged Gazebo topics /model/medibot/cmd_vel and /model/medibot/odometry. A static TF publisher emits an identity map → odom transform so RViz and the Path message use a consistent frame.

### 4.3.5 Caregiver Notification Module — caregiver_notify

The caregiver_notify node (ros2_ws/src/caregiver_notify/caregiver_notify/caregiver_notify_node.py) bridges emergency alerts and robot status to a caregiver notification channel, implementing the top tier of the three-layer architecture outlined in Chapter Three §3.4.

The node: - Subscribes to /emergency_alert and /robot_status - Publishes JSON payloads on /caregiver_notify (alert notifications) and /caregiver_status (robot progress updates) - Implements a cooldown mechanism (configurable, default 10 s) to avoid alert fatigue from repeated re-planning events

The caregiver notification payload structure:

{
  "caregiver_id": "ward-a",
  "type": "alert",
  "timestamp": 1759230432.131,
  "message": "Fall detected at patient_b (12, 9.5). Robot dispatched."
}

This decouples the caregiver interface from the reactive planning loop, permitting future extensions such as SMS/email dispatch, mobile app push, or two-way acknowledgment protocols without modifying the planning stack.

### 4.3.6 Dynamic Obstacle Module — human_obstacles

The human_obstacles node (ros2_ws/src/human_obstacles/human_obstacles/human_obstacles_node.py) translates animated human actors from Gazebo into dynamic obstacle poses for the planner. This module closes the loop on the three-layer system: IoT sensors detect an emergency (layer 1), caregiver is notified (layer 3), and the robot navigates through live human obstacles (layer 2).

The node: - Subscribes to actor pose updates from Gazebo (bridged via ros_gz_bridge on a scenario-specific topic /world/hospital_<scenario>/pose/info) - Filters poses to favour actor-height entities (z ≥ 0.9 m) and rejects NaN values - Publishes /dynamic_obstacles as a geometry_msgs/PoseArray with filtered actor poses - Is parameterized by actor_pose_topic, frame_id, and obstacle_radius

The path planner consumes /dynamic_obstacles and maintains a separate set of human-driven obstacles alongside event-triggered replans, allowing both stochastic planning strategies (A* and RRT*) to adapt to live crowd motion.

### 4.3.7 Logging and Evaluation Module — response_logger

The response_logger node writes two files into ~/fyp_ws/logs/:

- response_events.csv — one row per ROS event, with sim-time stamp, topic name, and the full JSON payload. This is the audit trail for debugging individual trials.

- trial_results.csv — one row per trial (per alert_id) with the schema:

alert_id, scenario, planner, run_id,
t_alert_s, t_plan_recv_s, t_arrived_s,
response_time_s, path_length_m, success,
compute_time_ms, replans

The logger maintains an in-memory dict keyed by alert_id. As alert, route, status, and caregiver messages arrive it merges their fields into the appropriate row and flushes a complete row to disk on either arrival/failure or after arrival_timeout_s (90 s) has elapsed since the alert. On flush it touches a sentinel file ~/fyp_ws/logs/run_<run_id>.done that the experiment orchestrator polls to know the trial is complete; this avoids race conditions between the launch-file teardown and the CSV append.

### 4.3.8 Launch / Coordination Module — emergency_launch

The launch file emergency_sim.launch.py exposes the trial-level parameters (planner_mode, scenario, run_id, seed, headless) and wires together: gz sim (with the scenario world), the parameter bridge for medibot control, the actor-pose bridge (scenario-specific), the static TF publisher, RViz 2 (in GUI mode only), and the six custom nodes: - emergency_publisher (sensor/alert) - path_planner (planning + human obstacle integration) - robot_controller (navigation + stall detection) - human_obstacles (actor pose → dynamic obstacle filter) - caregiver_notify (alert & status broadcast) - response_logger (CSV event recording)

## 4.4 Integration Workflow

Figure 4.1 shows the full ROS 2 graph. The data flow is:

- emergency_publisher waits 4 s for Gazebo to settle, then publishes one /emergency_alert message.

- path_planner subscribes to /emergency_alert, computes a route with the configured planner, and publishes the result on /planned_route.

- robot_controller consumes /planned_route, materialises it as a nav_msgs/Path on /planned_path (for RViz), and starts driving.

- gz sim consumes /model/medibot/cmd_vel (bridged from /cmd_vel), physically simulates the diff-drive robot, and publishes /model/medibot/odometry, which is bridged back to /odom.

- If the robot stalls (e.g. a moving actor blocks the corridor), the controller fires /replan_request; the planner injects a transient obstacle and republishes /planned_route.

- On arrival or final failure the controller publishes /robot_status with the final t_arrived_ns and replans count.

- response_logger joins /emergency_alert, /planned_route, and /robot_status on alert_id, writes the per-trial row, and drops the sentinel.

Figure 4.1. Node-and-topic graph of the emergency-response stack. Generated with rqt_graph. (file: data/figures/rqt_graph.png — capture during a Day 5 trial run)

## 4.5 Scenario Configuration

Three crowding scenarios are defined as YAML files under ros2_ws/src/emergency_launch/scenarios/:

| Scenario | Moving humans | Dynamic-event budget | World file |
| --- | --- | --- | --- |
| low_crowding | 2 actors @ 0.4 m/s | 2 | hospital_low_crowding.sdf |
| moderate_crowding | 4 actors @ 0.6 m/s | 4 | hospital_moderate_crowding.sdf |
| high_crowding | 7 actors @ 0.8 m/s | 7 | hospital_high_crowding.sdf |

All three share the same wall and bed layout (a 24 m × 14 m floor with a nurse station, a corridor, and three patient rooms) and the same three patient goal locations adjacent to each bed. The actor_count field of each scenario YAML matches the number of <actor> blocks in the corresponding world SDF, generated by worlds/_generate_scenario_worlds.py.

Figure 4.2. Plan view of the hospital world. (file: data/figures/world_overview.png — top-down RViz screenshot)

## 4.6 Execution and Sample Runs

### 4.6.1 Building and launching

source /opt/ros/jazzy/setup.bash
cd ~/Documents/<your-repo>/fyp_execution_kit
chmod +x build_and_test.sh
./build_and_test.sh
ros2 launch emergency_launch emergency_sim.launch.py \
   scenario:=moderate_crowding planner_mode:=a_star

### 4.6.2 Single-run trace

A representative single-trial trace from response_events.csv (moderate crowding, A*):

t_alert_s        topic            alert_id               replans  outcome
1759230432.131   emergency_alert  run17_alert1_84321     0        published
1759230432.155   planned_route    run17_alert1_84321     0        success, |path|=10
1759230445.802   replan_request   run17_alert1_84321     1        stall near (12,7)
1759230445.821   planned_route    run17_alert1_84321     1        success, |path|=12
1759230469.402   robot_status     run17_alert1_84321     1        arrived

Figure 4.3. RViz visualisation of an A* path through moderate crowding (sample from Day 5 trial). (file: data/figures/rviz_a_star_moderate.png)

Figure 4.4. RViz visualisation of an RRT* path through the same alert; note the more circuitous route consistent with sampling-based planning. (file: data/figures/rviz_rrt_star_moderate.png)

## 4.7 Implementation Challenges and Solutions

The following issues surfaced during integration and were resolved before the experiment matrix was run.

- Missing colcon resource markers. Six of the packages were missing their resource/<pkg> index files and Python __init__.py modules. colcon build rejected them with cryptic “package_index resource not found” errors. Fix: added empty marker files for each package and the missing __init__.py modules.

- Bridge / controller topic mismatch. The bridge exposed /model/medibot/cmd_vel and /model/medibot/odometry, but the controller used /cmd_vel and /odom. Symptom: the robot received no commands and stayed stationary. Fix: added explicit remappings= on the robot_controller Node entry in the launch file.

- Planner grid did not match the world. The original planner had a hand-coded 10 × 10 grid with arbitrary obstacles unrelated to the SDF walls, so it routinely returned paths through walls. Fix: introduced the scenario_file parameter and loaded planner_grid.obstacles from YAML, with the grid hand-mirrored from the SDF wall + bed coordinates.

- No map → odom transform. Without a TF link RViz refused to render the planned Path. Fix: added a tf2_ros static_transform_publisher emitting an identity transform from map to odom.

- RRT* failure rate in dense scenarios. The lightweight RRT* prototype (in scripts/run_scenario.py) had failed in moderate and high crowding. The ROS 2 implementation initially did the same. Fix: raised rrt_max_iterations to 2000, dropped rrt_step_size to 0.8, and raised rrt_goal_sample_rate to 0.25; success-rate jumped above acceptable thresholds (Chapter Five §5.3).

- Animated actors required a model-cache populated from Gazebo Fuel. <actor> blocks reference the standard walk.dae skin from fuel.gazebosim.org. Resolved by running the launch once with internet, after which the model is cached under ~/.gz/fuel/. A static fallback set of cylinder humans is documented in the README for offline use.

- Actor pose bridge topic naming. The Gazebo actor pose topic varies by world name (e.g., /world/hospital_low_crowding/pose/info vs /world/hospital_high_crowding/pose/info). Fix: made the actor-pose topic name scenario-dependent in the launch file using ConcatSubstitution, and the human_obstacles node accepts it as a configurable parameter.

- Human obstacle filtering false positives. Initially, the human_obstacles node subscribed to all Gazebo entities and mapped them blindly onto the planner grid, including static models and light sources. This caused spurious obstacles to appear mid-path. Fix: added a z-height filter (min z ≥ 0.9 m) and NaN rejection to isolate actor-like entities.

- Caregiver notification cooldown interaction with replans. Early versions published one caregiver notification per replan event, causing alert spam. Fix: implemented a per-caregiver cooldown (default 10 s) so only the first alert and status changes at major milestones trigger notifications, reducing false-positive urgency signals.

## 4.8 Chapter Summary

The six-package ROS 2 stack runs end-to-end in Gazebo Sim Harmonic, implementing the full three-layer emergency-response architecture:

- Layer 1 (IoT Sensing): emergency_publisher publishes /emergency_alert from a simulated wearable with patient location and severity.

- Layer 2 (Robot Response): path_planner and robot_controller compute and execute routes, consuming /dynamic_obstacles from human actors and triggering replans on stall events.

- Layer 3 (Caregiver Notification): caregiver_notify broadcasts alerts and status to /caregiver_notify and /caregiver_status with cooldown anti-spam measures.

Supporting modules: - human_obstacles: bridges animated actor poses to dynamic obstacle grid cells. - response_logger: joins alert, route, and status messages on alert_id and writes per-trial CSV rows for evaluation.

An emergency alert flows from the simulated wearable through the planner, the controller, and the bridged Gazebo robot, with stall detection driving a replan loop that integrates both event-triggered obstacles (transient) and human-driven obstacles (continuous). Every trial produces a per-row CSV entry capturing all timestamps needed to compute response time, planning time, path length, replan count, and success status. The system is now ready for the quantitative comparative evaluation reported in Chapter Five.

## Appendix B – Reproducibility

sudo apt update
sudo apt install -y \
   ros-jazzy-ros-base \
   ros-jazzy-ros-gz-bridge \
   ros-jazzy-ros-gz-sim \
   ros-jazzy-rviz2 \
   ros-jazzy-tf2-ros \
   ros-jazzy-nav-msgs \
   ros-jazzy-geometry-msgs \
   python3-colcon-common-extensions \
   python3-rosdep \
   python3-yaml
sudo rosdep init || true
rosdep update
mkdir -p ~/fyp_ws/src
cp -r fyp_execution_kit/ros2_ws/src/. ~/fyp_ws/src/
cd ~/fyp_ws
rosdep install --from-paths src --ignore-src -r -y
colcon build --symlink-install
source install/setup.bash
ros2 launch emergency_launch emergency_sim.launch.py